MUDPRO+ Formula Reference

Codebase snapshot reviewed on July 6, 2026. This document summarizes the formulas currently implemented across the desktop app and backend.

Use this as a technical reference for report calculations, engineering helpers, pit balancing, survey math, hydraulics, and inventory-volume conversions.

1. Scope And Conventions

| Calculation | Formula, Notes, And Source |
| --- | --- |
| Document scope | Formula: This document covers the calculation logic currently implemented in the MudPro desktop app and backend services. Notes: It focuses on engineering, report, pit-volume, operation, hydraulics, survey, concentration, and solids-analysis formulas. CRUD-only screens and pure unit-conversion tables are excluded. Source: MudPro-Desktop-App/lib, Mud-pro-Backend/src |
| Rounding | Formula: Most report-volume values use round2(value) = Number(value.toFixed(2)); some nozzle and product values use 3 or 4 decimals. Notes: UI displays usually follow bbl = 2-3 decimals, nozzle diameter = 4 decimals, pump displacement = 4 decimals, pump rate = 1 decimal. Source: volumeName.controller.js, nozzle.controller.js, pump.controller.js, consumeProductController.js |
| Base units | Formula: Core formulas are evaluated in base oilfield units such as ft, in, bbl, gpm, psi, ppg, and lb/ft before AppUnits converts them for display. Notes: This is why many UI screens first convert user input to a fixed base unit and convert the result back to the selected unit system. Source: AppUnits usage across well_tab_content.dart, pump_tab_content.dart, formation_controller.dart, engineering_tools_controller.dart, bit_hydra_controller.dart |

2. Well, Hole, And Pipe Geometry

| Calculation | Formula, Notes, And Source |
| --- | --- |
| Raw cylinder volume | Formula: Volume_bbl = (ID_in^2 x Length_ft) / 1029.4 Notes: This is the base cylinder formula used for pipe IDs, pipe ODs, casing IDs, and hole intervals. Source: pitvolumename/volumeName.controller.js rawCylinderVolume(), Export/exportInventoryController.js calculatePipeVolume() |
| Cased-hole length | Formula: Length_ft = abs(Shoe - Top) when both are valid; otherwise max(0, min(MD, Shoe) - Top) or max(0, MD - Top) Notes: The code supports full casing rows and liner rows. Source: pitvolumename/volumeName.controller.js casedHoleLength() |
| Cased-hole raw volume | Formula: CasedHole_bbl = sum(rawCylinderVolume(ID, Length)) across valid casing rows Notes: If liner rows exist, the controller also adds the previous casing volume up to the first liner top. Source: pitvolumename/volumeName.controller.js calculateCasedHoleRawVolume() |
| Open-hole volume with washout | Formula: EffectiveHoleID_in = HoleID_in x (1 + Washout_pct / 100) Formula: OpenHole_bbl = sum(rawCylinderVolume(EffectiveHoleID, MD_i - PreviousDepth)) Notes: PreviousDepth starts at the deepest cased-hole shoe. Source: pitvolumename/volumeName.controller.js calculateOpenHoleRawVolume() |
| Drill-string steel and inside volume | Formula: PipeSteel_bbl = sum(rawCylinderVolume(OD, PieceLength)) Formula: PipeInside_bbl = sum(rawCylinderVolume(ID, PieceLength)) Notes: PieceLength is clipped to the depth limit, usually bit depth or MD. Source: pitvolumename/volumeName.controller.js calculateDrillStringGuideVolumes() |
| Displacement and hole volume | Formula: HoleSpace_bbl = CasedHole_bbl + OpenHole_bbl Formula: Displacement_bbl = max(0, PipeSteel_bbl - PipeInside_bbl) Formula: Hole_bbl = HoleSpace_bbl - PipeSteel_bbl + PipeInside_bbl Notes: The report uses Hole as the visible fluid-holding capacity after string displacement. Source: pitvolumename/volumeName.controller.js calculateCombinedHoleVolumeResult() |
| Annulus in pit snapshot | Formula: Annulus_bbl = max(0, Hole_bbl - DrillStringInside_bbl) Notes: DrillStringInside is the counted pipe internal volume from the hole-volume result. Source: pitvolumename/volumeName.controller.js lines building pit snapshot payload |
| Top plug | Formula: Capacity_bbl_per_ft = 0.0009714 x HoleDiameter_in^2 Formula: PlugTop_ft = max(MD_ft - (CementPlugVol_bbl / Capacity_bbl_per_ft), 0) Notes: UI converts the entered volume and length into bbl and ft before calculating. Source: dashboard/tabs/well_tab_content.dart _calculateTopPlug() |
| Drill-string ID from OD and weight | Formula: ID_in = sqrt(OD_in^2 - (Weight_lb_per_ft / 2.672)) Notes: Used by the report Well tab helper when OD and linear weight are known. Source: dashboard/tabs/well_tab_content.dart _calculateIds() |

3. Nozzle And Bit Calculations

| Calculation | Formula, Notes, And Source |
| --- | --- |
| Single nozzle geometry | Formula: Diameter_in = Size_32nds / 32 Formula: Area_in2 = (pi x Diameter_in^2) / 4 Notes: Backend stores both diameter and area per processed row. Source: controllers/nozzle/nozzle.controller.js calculateNozzleArea() |
| Total flow area (TFA) | Formula: TFA_in2 = sum(RawArea_in2 x Count) across all valid nozzle rows Notes: The frontend nozzle controller mirrors the backend formula locally. Source: controllers/nozzle/nozzle.controller.js processNozzles(), dashboard/controller/nozzle_controller.dart recalculateTfa() |
| Bit hydraulics utility | Formula: TotalJetArea_in2 = sum(0.785 x Dia_in^2) Formula: NozzleVelocity_ft_per_s = (0.408 x Q_gpm) / TotalJetArea Formula: BitPressureDrop_psi = 0.65 x StandpipePressure_psi Formula: HydraulicHP = (BitPressureDrop_psi x Q_gpm) / 1714 Formula: BitArea_in2 = 0.785 x BitSize_in^2 Formula: HHP_per_Area = HydraulicHP / BitArea Formula: PressureDrop_pct = (BitPressureDrop / StandpipePressure) x 100 Formula: JetImpact_lbf = 0.01823 x MW_ppg x Q_gpm x NozzleVelocity Source: utility/controller/bit_hydra_controller.dart calculateBitHydraulics() |

4. Pump, Circulation, And Hydraulics

| Calculation | Formula, Notes, And Source |
| --- | --- |
| Pump displacement | Formula: Duplex with rod = 0.000162 x (2 x D^2 - d^2) x L x Eff Formula: Duplex without rod = 0.000324 x D^2 x L x Eff Formula: Triplex = 0.000243 x D^2 x L x Eff Formula: Quadplex = 0.000324 x D^2 x L x Eff Formula: Quintuplex = 0.000405 x D^2 x L x Eff Notes: Eff is efficiency expressed as a fraction, not percent. Source: controllers/pump/pump.controller.js calculateDisplacement(), UG/model/pump_model.dart recalculateDisplacement() |
| Pump rate | Formula: Rate_gpm = Displacement_bbl_per_stk x SPM x 42 Notes: Both backend and report UI use the same oilfield conversion factor. Source: controllers/pump/pump.controller.js calculateRate(), dashboard/tabs/pump_tab_content.dart recalculateRate() |
| Circulation timing | Formula: Strokes = Volume_bbl / Displacement_bbl_per_stk Formula: Minutes = (Volume_bbl x 42) / PumpRate_gpm Fallback: Minutes = Strokes / StrokePumpSPM when pump rate is unavailable Source: Export/exportInventoryController.js calculateCirculationTiming() |
| Annular velocity utility | Formula: AV_ft_per_min = (24.51 x Q_gpm) / (Dh_in^2 - Dp_in^2) Notes: The calculator first converts flow to gpm and diameters to inches. Source: utility/controller/engineering_tools_controller.dart calculateAnnularVelocity() |
| Hydraulic ECD | Formula: ECD_ppg = MW_ppg + (PressureLoss_psi / (0.052 x Depth_ft)) Notes: If pressure loss or depth is missing, the result falls back to base mud weight. Source: Export/exportInventoryController.js calculateEcd(), report/recap hydra controller _calculateEcd() |
| Critical velocity | Formula: Gap_in = HoleSize_in - PipeOD_in Formula: CriticalVelocity = (1.08 x PV + 1.08 x sqrt(PV^2 + 12.34 x Gap^2 x YP x MW)) / (MW x Gap) Source: Export/exportInventoryController.js hydraulicCriticalVelocity() |
| Pipe and annular velocity | Formula: PipeVelocity_ft_per_min = (24.51 x Q_gpm) / ID_in^2 Formula: DrillStringVelocity_ft_per_s = PipeVelocity_ft_per_min / 60 Formula: AnnularVelocity_ft_per_min = (24.51 x Q_gpm) / (Dh_in^2 - OD_in^2) Source: Export/exportInventoryController.js hydraulicPipeVelocity(), hydraulicDrillStringVelocity(), hydraulicAnnularVelocity() |
| Annular and drill-string pressure loss | Formula: AnnularLoss = (PV x L x AV) / (60000 x Gap^2) + (YP x L) / (225 x Gap) Formula: DrillStringLoss = (PV x L x PipeVel) / (PipeDenominator(ID) x ID^2) + (YP x L) / (225 x ID) Notes: PipeDenominator(ID) is a polynomial derived from the legacy hydraulics model. Source: Export/exportInventoryController.js hydraulicAnnularPressureLoss(), hydraulicDrillStringPressureWeight(), hydraulicPipePressureDenominator() |
| Bit pressure loss and jet velocity | Formula: BitLoss_psi = (MW_ppg x Q_gpm^2) / (10858 x TFA_in2^2) Formula: JetVelocity_ft_per_s = (0.32086 x Q_gpm) / TFA_in2 Source: Export/exportInventoryController.js hydraulicBitPressureLoss(), hydraulicBitJetVelocity() |

5. Mud, Rheology, Solids, And Formation Windows

| Calculation | Formula, Notes, And Source |
| --- | --- |
| Bingham rheology | Formula: PV_cP = R600 - R300 Formula: YP_lbf_per_100ft2 = R300 - PV Source: dashboard/controller/mud_controller.dart calculateRheology() |
| Power Law rheology | Formula: n = 3.32 x log10(R600 / R300) Formula: K = R600 / (1022^n) Source: dashboard/controller/mud_controller.dart calculateRheology() |
| Herschel-Bulkley rheology | Formula: YieldStress = max(0, 2 x R3 - R6) Formula: Adjusted600 = R600 - YieldStress Formula: Adjusted300 = R300 - YieldStress Formula: n = 3.32 x log10(Adjusted600 / Adjusted300) Formula: K = Adjusted600 / (1022^n) Source: dashboard/controller/mud_controller.dart calculateRheology() |
| Formation window conversions | Formula: Gradient_psi_per_ft = PPG x 0.052 Formula: Pressure_psi = Gradient x TVD_ft Formula: PPG = Gradient / 0.052 Formula: Gradient = Pressure / TVD_ft Source: UG/controller/formation_controller.dart _recalculateGroup() |
| Solids analysis - brine and corrected solids | Formula: BrineSG = 0.99707 + 0.007923 x CaCl2_pct + 0.00004964 x CaCl2_pct^2 Formula: BrineVol_pct = (100 x Water_pct) / (BrineSG x (100 - CaCl2_pct) x 0.99707) when salt correction is needed Formula: DissolvedSolids_pct = max(0, (BrineSG - 1) x 100) Formula: CorrectedSolids_pct = 100 - (Oil_pct + BrineVol_pct) unless directly provided Source: controllers/SolidAnalysis/solidanalysiscontroller.js computeSolidsAnalysis() |
| Solids analysis - average density and phase split | Formula: AvgSolidsSG = (100 x (MW / 8.34) - Oil_pct x OilSG - BrineSG x BrineVol_pct) / CorrectedSolids_pct Formula: HGS_pct = ((AvgSolidsSG - LGS_SG) / (HGS_SG - LGS_SG)) x CorrectedSolids_pct Formula: LGS_pct = CorrectedSolids_pct - HGS_pct Formula: LGS_lb_per_bbl = 3.5 x LGS_SG x LGS_pct Formula: HGS_lb_per_bbl = 3.5 x HGS_SG x HGS_pct Source: controllers/SolidAnalysis/solidanalysiscontroller.js computeSolidsAnalysis() |
| Solids analysis - bentonite and drill solids | Formula: Bentonite_pct = Bentonite_lb_per_bbl / (3.5 x 2.65) Formula: DrillSolids_pct = LGS_pct - Bentonite_pct Formula: DrillSolids_lb_per_bbl = 3.5 x LGS_SG x DrillSolids_pct Formula: DS_Bent_Ratio = DrillSolids_pct / Bentonite_pct Formula: TotalSolids_lb_per_bbl = MW x 42 x (TotalSolids_pct / 100) Source: controllers/SolidAnalysis/solidanalysiscontroller.js computeSolidsAnalysis() |

6. Operations And Inventory Calculations

| Calculation | Formula, Notes, And Source |
| --- | --- |
| Consume Product row math | Formula: Final = Initial + Received - Returned - Adjust - Used Formula: Cost = Used x Price Formula: Volume_bbl is calculated only when the selected product is flagged for volume addition Source: dashboard/tabs/operation/consume_product.dart ProductRowData.recalculate() |
| Consume Product unit-to-volume conversion | Formula: gal -> bbl = TotalUnits / 42 Formula: bbl -> bbl = TotalUnits Formula: kg -> bbl = TotalUnits / (SG x 158.987) Formula: lb -> bbl = TotalUnits / (SG x 350) Formula: ton or mt -> bbl = (TotalUnits x 2000) / (SG x 350) Formula: liter -> bbl = TotalUnits / 158.987 Formula: ml -> bbl = TotalUnits / 158987 Formula: m3 -> bbl = TotalUnits x 6.28981 Source: controllers/Consumeproduct/consumeProductController.js calculateVolumeBbl(), dashboard/tabs/operation/consume_product.dart _calculateVolumeBbl() |
| Consume Product total volume | Formula: TotalVol = sum(ProductCalculatedVolume) + WaterVolume_if_AddWaterChecked Notes: This is the value shown in the Add Water panel and used for automatic Active System distribution. Source: dashboard/tabs/operation/consume_product.dart _recalculateTotalVolume() |
| Distribution behavior | Formula: If no manual distribution rows exist, DistributeRow[0].Volume = TotalVol Notes: This preserves the legacy Active System mirroring behavior. Source: dashboard/tabs/operation/consume_product.dart _rebalanceDistributeVolumes() |
| Receive Mud | Formula: NetVolume = GrossVolume - LossVolume Notes: MW, mud type, and leasing fee fall back to the selected premixed mud master when blank. Source: controllers/receivemud/receiveMud.controller.js prepareReceiveMudData() |
| Return / Lost Mud | Formula: TotalDeduct = VolReturned + VolLost Notes: Source availability is validated before the transaction is accepted. Source: controllers/returnlostmud/returnLostMud.controller.js prepareReturnLostMudData(), assertSourceHasAvailableVolume() |
| Other Volume Addition | Formula: TotalVolume = Formation + Cuttings + VolumeNotFluid Source: controllers/othervol/otherVolAddition.controller.js prepareOtherVolAdditionData() |
| Mud Loss - Active System | Formula: TotalLoss = CuttingsRetention + Seepage + Dump + Shakers + Centrifuge + Evaporation + PitCleaning + Formation + AbandonInHole + LeftBehindCasing + Tripping + ExtraLoss Notes: The backend validates that the Active System has enough available volume before accepting the loss. Source: controllers/mudloss/mudLoss.controller.js createMudLoss() |
| Consume Service / Package / Engineering | Formula: PackageFinal = Initial - Used Formula: PackageCost = Used x Price Formula: ServiceCost = Usage x Price Formula: EngineeringCost = Usage x Price Source: dashboard/tabs/operation/consume_service.dart |
| Cuttings/Retention helper dialog | Formula: CuttingsRetention_bbl = VolDrilled_bbl x MudLossRatio_pct / 100 Source: dashboard/tabs/operation/operation_desktop_ui.dart showCuttingsRetentionDialog() |
| Evaporation helper dialog | Formula: ActiveHours = DrillingHours + CirculatingHours Formula: Evaporation_bbl = max(FlowlineTemp_F - 32, 0) x ActiveHours / 1000 Source: dashboard/tabs/operation/operation_desktop_ui.dart showEvaporationDialog() |
| Other Volume Addition helper dialog | Formula: CuttingsGain = VolDrilled when ShakerBypass = Yes Formula: CuttingsGain = VolDrilled x (1 - Efficiency_pct / 100) when ShakerBypass = No Source: dashboard/tabs/operation/othervolumeaddition_view.dart |

7. Pit, Volume Name, And Ledger Balancing

| Calculation | Formula, Notes, And Source |
| --- | --- |
| Active Pits total | Formula: ActivePits_bbl = sum(ActivePitMeasuredVolume) Source: pitvolumename/volumeName.controller.js active pit reduction and snapshot builders |
| Active System | Formula: ActiveSystem_bbl = Hole_bbl + ActivePitsWithTransfer_bbl Notes: ActivePitsWithTransfer includes transfer deltas and consume-product distribution deltas by active pit. Source: pitvolumename/volumeName.controller.js derivedActiveSystem |
| End volume | Formula: EndVol_bbl = CarryOverBaseline + OperationLedgerDelta Formula: OperationLedgerDelta = EndVolDelta + DistributedToActiveSystem + DistributedToActivePits Exception: EndVol = 0 when the report has no current volume data and no carry-over, or when Empty Active System uses Dump. Source: pitvolumename/volumeName.controller.js calculateEndVolForReport() |
| Pending difference | Formula: Pending_bbl = EndVol_bbl - ActiveSystem_bbl Notes: This is the 'End Vol. - Active System' value shown to the user for pit adjustment. Source: pitvolumename/volumeName.controller.js endVolMinusActiveSystem |
| Total on location | Formula: TotalOnLocation_bbl = ActiveSystem_bbl + AdjustedTotalStorage_bbl Formula: AdjustedTotalStorage_bbl = TotalStorage_bbl + ReturnLostStorageDelta_bbl Source: pitvolumename/volumeName.controller.js totalOnLocation |
| Ledger-style total on location | Formula: LedgerTotal = ConsumeProductTotal + ReceivedMudTotal + AddWaterTotal + OtherVolAdditionTotal - LostMudTotal - MudLossTotal - MudLossStorageTotal Notes: This is kept as a separate derived total in the pit snapshot payload. Source: pitvolumename/volumeName.controller.js ledgerTotalOnLocation |
| Operation deltas that affect End Vol | Formula: Add Water to Active System or active pit -> EndVolDelta += Volume Formula: Receive Mud to Active System or active pit -> EndVolDelta += NetVolume Formula: Return/Lost from Active System or active pit -> EndVolDelta -= (Returned + Lost) Formula: Other Vol Addition -> EndVolDelta += TotalVolume Formula: Mud Loss Active System -> EndVolDelta -= TotalLoss Formula: Transfer from active to storage -> EndVolDelta -= TransferVolume Formula: Transfer from storage to active -> EndVolDelta += TransferVolume Formula: Empty Active System -> Transfer to Storage subtracts from EndVol; Dump forces EndVol to zero Source: pitvolumename/volumeName.controller.js buildOperationVolumeEffects() |

8. Survey, Concentration, And Cost Splits

| Calculation | Formula, Notes, And Source |
| --- | --- |
| Survey minimum-curvature dogleg | Formula: cos(Dogleg) = cos(Inc1) x cos(Inc2) + sin(Inc1) x sin(Inc2) x cos(Azi2 - Azi1) Formula: RatioFactor = 1 when Dogleg ~= 0 else (2 / Dogleg) x tan(Dogleg / 2) Source: UG_ST_navigation/.../survey_controller.dart _recalculateAllRows() |
| Survey TVD, North, East | Formula: dTVD = (dMD / 2) x (cos(Inc1) + cos(Inc2)) x RatioFactor Formula: dNorth = (dMD / 2) x (sin(Inc1)cos(Azi1) + sin(Inc2)cos(Azi2)) x RatioFactor Formula: dEast = (dMD / 2) x (sin(Inc1)sin(Azi1) + sin(Inc2)sin(Azi2)) x RatioFactor Formula: TVD, North, and East are cumulative sums of their interval deltas Source: UG_ST_navigation/.../survey_controller.dart _recalculateAllRows() |
| Survey vertical section and DLS | Formula: VSec = sqrt(North^2 + East^2) when ProjectAzimuth is disabled Formula: VSec = North x cos(ProjectAzimuth) + East x sin(ProjectAzimuth) when enabled Formula: DoglegSeverity = Dogleg_deg x IntervalBase / dMD, where IntervalBase is 100 ft or 30 m by unit setting Source: UG_ST_navigation/.../survey_controller.dart _calculateVerticalSection() |
| Recap concentration | Formula: Concentration = (Quantity x FactorPerPack) / SystemVolume Formula: ton factor -> amount x 2000 lb; kg factor -> amount x 2.20462 lb; gal factor -> amount gal Notes: Output unit depends on pack class, usually lb/bbl or gal/bbl. Source: report/recap_tabs/concentration/controller/recap_concentration_controller.dart |
| Daily cost percentages | Formula: PercentShare = (GroupAmount / TotalAmount) x 100 Source: daily_report/tabs/daily_cost/tabs/dailycost_percentagetable.dart |

9. Usage Notes

If a displayed value does not match expectations, verify the active report scope, carry-over source, unit system, and whether the UI is showing calculated volume, measured volume, or pending adjustment volume.

Several recap, export, and pit-snapshot screens reuse the same backend formulas. When two screens disagree, the scope and source data usually differ before the formula does.